# Sequencing steps

- Big Dye PCR

- G50 clean PCR

- Dry sample

- Load gel

# Big Dye PCR

1/4 reaction	Big Dye	2.0   ul		Cycle		96 C 	10 secs

Primer 10uM	0.33 ul				50 C	  5 secs

5x buffer	1.0   ul				60 C	  4 mins

Template	1-2   ul				40 cycles

Water	to 	10    ul

#### Sephadex G50 columns

Resuspend 1 gram G50 in 16 ml good water.

Let rehydrate for at least 60 minutes.

Dispense 0.8 ml rehydrated G50 into Centri-Sep Column.

Spin 2 minutes at 700g.

Transfer column into new microfuge tube.

Add sample to sloped surface.

Spin 2 minutes at 700g.

### Centri-Sep Spin columns – Princeton Separations CS-900

Dry sample in speed-vac

Suspend dry sample in 3-5ul formamide:sequencing dye (5:1, 100ul:20ul)

## Seq Dye		25mM 		EDTA pH 8.0

50mg/ml	Blue Dextran MW 2,000,000

Heat samples 95C for 3 minutes

Pour / Load Gel

## Prepare gel		1 	Magnetic Stir Bar

18g 	Urea

5ml	Long ranger acrylamide

10ml 	5x TBE

21ml	DD Water

Polymerize with 	125ul APS		250ul

17ul TEMED		  35ul

Pre-run gel 30 minutes, 1x TBE

Load 2-3ul sample in 5:1 dye – Run gel.

Module48E-1200	2400 	volts

50	mAmp

200	watts

51oC

40 	mWatt Laser

10 	hours