# Sephadex G50 columns

Resuspend 1 gram G50 in 16 ml good water.

Let rehydrate for at least 60 minutes.

Dispense 0.8 ml into Centri-Sep Column.

Spin 2 minutes at 700g.

Transfer column into new microfuge tube.

Add sample to sloped surface.

Spin 2 minutes at 700g.

Dry sample in speed-vac.

Centri-Sep Spin columns – Princeton Separations CS-900